XML Validator User Guide

# Overview

The Validator tool is a small application, which helps XML file design by:

- checking GenICam rules not enforced by the schema

- checking tree nodes consistency

- providing a graphic view of a node and its dependencies

- importing node and dependent nodes from another xml file

- small edition capabilities

It does not replace XML tool's edition, xml syntax checker, grammar checker capabilities, but works as a complement of these tools.

XML file validity is proven if:

- the XML file syntax is valid (using an XML editor or any explorer to display it)

- the XML file GenICam grammar is valid (using an XML editor, with grammar check capabilities)

- the GenICam Validator does not report any error

# Features

The XML file is represented as a node tree, with the following pictograms:

This is a XML comment

This is a XML element

This is a XML element's content

## Load XML file

Load the XML file and convert it into a node tree. At this point, if the XML file syntax is not valid, the error is reported into the status area and the load process stops.

## Save XML file

Save the node tree into a GenICam XML file.

## Check XML validity

The check reports in the status area the different error. A double click on an error selects the erroneous nodes in the tree node.

The check is able to detect:

- node pointing to unavailable node (unavailable reference)

- node not referenced by another node (un-referenced node)

- node name conflict (same node name) : all the conflict nodes appears in the status area

- first category name which is not "Root" (not enforced by the schema)

- additional occurrence constraint not respected (not enforced by the schema)

## Node edition

For all the nodes, move comment, element, element content by:

- CTRL – or Right click on node and "Move Up": to move node up

- CTRL + or Right click on node and "Move Down": to move node down

### XML comment

Insert a comment by right click on a node, selecting menu New/Comment. The comment is inserted at the end of the selected node.

Delete a comment by right clicking on a node, selecting menu delete.

Modify a comment by double click on the comment.

### XML element (and attributes)

Insert an element by right click on a node, selecting menu New/Element. The inserted element is added at the end of the selected node.

Delete an element by right clicking on a node, selecting menu delete.

Modify an element by double click on the element. The edit mode appears (Enter or ESC to close it)

This edit mode allows you:

- to change the element name

- to add, delete, move attributes for the element

- to set the attribute value

### XML element content

Insert an element content by right click on a node, selecting menu New/Text. The inserted element content is added at the end of the selected node.

Delete an element content by right clicking on a node, selecting menu delete.

Modify an element content by double click on the element content.

## Nodes import

To import a set of XML nodes from another XML file, the drag and drop features is supported between two Validator applications.

Open a second Validator and load the XML source file. In this second Validator, select a  node, drag it over the first Validator, and drop it anywhere.

The selected node and all its dependencies are now imported at the end of the first Validator. You will probably have to resolve name clash conflict.

## Node graph generation

The Validator is able to generate a graph for a node and its dependencies. The graph generation is achieved by Graphwiz, so Graphwiz runtime must be available. The generated graph is viewed using your default image viewer.

Generate a graph by right click on a node, selecting menu print. The following print dialog opens:

This dialog requires:

- The dot.exe path (graphviz can be downloaded from http://www.graphviz.org/)

- The output file name

- The output file type

- The graph generation depth

The graph generation stops when the number of node dependency level from the selected node is greater than the graph generation depth.

Be careful with the graph generation. Depending on your selected node, the generated graph can be huge and easily reach the GraphViz generation limitation or image viewer limitation.

The erroneous node are highlighted in red on the graph.

Graph sample